Application No.:  361714

Equipment Category – Aluminum Melting Furnace

| 1. GENERAL INFORMATION |  |  | DATE: 5/13/2003 |
| --- | --- | --- | --- |
| a. manufacturer: |  |  |  |
| b. type: Reverberatory |  | c. model: |  |
| d. style: Well-Charged |  |  |  |
| e. applicable aqmd rules: RECLAIM |  |  |  |
| f. cost: $ () source of cost data: |  |  |  |
| g. operating schedule: 24 hrs/day 7 days/wk 52 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: 361714 |
| --- | --- | --- | --- |
| A. FUNCTION: This is Furnace No. 4, Device No. D20, a RECLAIM Cycle 1 Large Source. This is a Title V facility. This furnace melts scrap aluminum (beverage cans) for a rolling mill. |  |  |  |
| B. maximum Heat Input: 31.5 MMBtu/hr |  | C. maximum throughput: 9.75 million lb per mo. (240,000 lb capacity) |  |
| D. burner information: no.: 2 type: Regenerative, Low-NOx |  |  |  |
| e. primary fuel: Natural Gas |  | f. other fuel: None |  |
| g. operating conditions: Continuous |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: 361714 |  |
| --- | --- | --- | --- | --- |
| A. NAME: Commonwealth Aluminum Concast |  |  |  | B. SIC CODE: 3355 |
| C. ADDRESS: 2211 East Carson Street CITY: Long Beach STATE: CA ZIP: 90810 |  |  |  |  |
| D. CONTACT PERSON: Kevin Conkright |  | E. PHONE NO.: 270-733-1212 |  |  |

| 4. PERMIT INFORMATION |  |  |  | App. No.: 361714 |
| --- | --- | --- | --- | --- |
| A. AGENCY: SCAQMD |  | B. application type: |  |  |
| C. AGENCY CONTACT PERSON: Augustine Agwuenu |  |  | D. PHONE NO.: 909-396-2632 |  |
| E. PERMIT TO CONSTRUCT/OPERATE INFORMATION: P/C NO.: ISSUANCE DATE: check if no p/c P/O NO.: 361714 ISSUANCE DATE: 3/8/2000 |  |  |  |  |
| F. START-UP DATE: April 1995 |  |  |  |  |

| 5. EMISSION INFORMATION |  |  | App. No.: 361714 |  |  |
| --- | --- | --- | --- | --- | --- |
| A. PERMIT |  |  |  |  |  |
| A1. PERMIT LIMIT: 60 ppmvd@3%O2 RECLAIM concentration limit. Mass emission limits (lb/month): 324 ROG, 770 CO, 583 PM, 14 SOx. |  |  |  |  |  |
| A2. bact/lAer DETERMINATION: Natural gas, low-NOx, high-efficiency burner |  |  |  |  |  |
| a3. BASIS OF THE BACT/LAER DETERMINATION: |  |  |  |  |  |
| ##### B. CONTROL TECHNOLOGY |  |  |  |  |  |
| B1. MANUFACTURER/SUPPLIER: North American Manufacturing Co. |  |  |  |  |  |
| B2. TYPE: Regenerative, low-NOx |  |  |  |  |  |
| B3. DESCRIPTION: Model No. 4343-12B, Twin-Bed burner with Low-NOx Injection (LNI). This burner is equipped with a fuel injector designed to reduce the air/fuel mixing rate, thus lowering peak flame temperature and NOx. The Twin-Bed burner is designed to capture heat from the exhaust gases and preheat the combustion air, thereby reducing fuel requirements in a manner similar to a regenerative thermal oxidizer. The furnace exhaust gases flow through a bed of ceramic material, transferring heat to the bed. Combustion air enters the burner through a second bed, which has been previously heated by contact with exhaust gases. The direction of flow is periodically reversed by valve actions so that the inlet bed becomes the outlet bed, and visa versa. These burners were installed in 1995 as replacement burners in order to enable the furnace to be reclassified from RECLAIM Major Source to RECLAIM Large Source, thus eliminating a need for continuous monitoring of NOx emissions. |  |  |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: ISSUANCE DATE: P/O NO.: ISSUANCE DATE: |  |  |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: ACTUAL CONTAMINANT LOADING: BLOWER HP: |  |  |  |  |  |
| B6. WARRANTY: |  |  |  |  |  |
| B7. PRIMARY POLLUTANTS: NOx, CO, VOC, PM10 |  |  |  |  |  |
| B8. SECONDARY POLLUTANTS: |  |  |  |  |  |
| B9. SPACE REQUIREMENT: |  |  |  |  |  |
| B10. LIMITATIONS: |  |  |  | ###### B11. UNUSED |  |
| B12. OPERATING HISTORY: The burners have functioned well since their startup in 1995. |  |  |  |  |  |
| ### B13. UNUSED |  | ### B14. UNUSED |  |  |  |
| ##### C. CONTROL EQUIPMENT COSTS |  |  |  |  |  |
| C1. CAPITAL COST: check if installation cost is included in EQUIPMENT cost EQUIPMENT: $ INSTALLATION: $ () SOURCE OF COST DATA: |  |  |  |  |  |
| C2. ANNUAL OPERATING COST: $ () SOURCE OF COST DATA: |  |  |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: DATE: |  |  |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: The furnace is required to be source tested every three years. |  |  |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: None DATES: CAUSES: |  |  |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: 3 related to furnaces DATES: 1998-1999 CAUSES: Smoke and odors from furnaces |  |  |  |  |  |
| D5. MAINTENANCE REQUIREMENTS: Ash is periodically drained from the bottom hoppers of the ceramic beds. Major maintenance is done during scheduled furnace maintenance outages, which occur twice per year. |  |  |  |  | D6. UNUSED |
| D7. SOURCE TEST/PERFORMANCE DATA RESULTS AND ANALYSIS: DATE OF SOURCE TEST: 12/18/1998, 11/7-8/2001 CAPTURE EFFICIENCY: DESTRUCTION EFFICIENCY: OVERALL EFFICIENCY: SOURCE TEST/PERFORMANCE DATA: Date 12/18/1998 11/7-8/2001 NOx, ppmvd@3%O2 46 50.1 O2, % (dry) 18.1 14.8 Fuel, scfm 125.0 444.5 OPERATING CONDITIONS: Normal. Tests were each one hour duration. High O2 values are mainly due to air being induced into the stack. TEST METHODS: AQMD Method 100.1. |  |  |  |  |  |

| 6. COMMENTS |  | App. No.: 361714 |
| --- | --- | --- |
| The facility has records showing that the fuel usage of Furnace No. 4 is 30 to 40% less than Furnace No. 1, which is identical except that it is equipped with cold-air burners. A reduction in fuel input results in an equal reduction in NOx emissions at a given NOx ppm level. Therefore, in terms of NOx emissions, the 60 ppm limit on Furnace No. 4 is equivalent to 36 to 42 ppm on a furnace with cold-air burners. Regenerative burners cost more than conventional burners and require more maintenance. The manufacturer has pointed out that they are best suited to relatively large, highly utilized furnaces. With regard to aluminum melting furnaces, the manufacturer recommends them for three furnace types: well-charged, direct-charged, and dry-hearth. Due to the potential for fouling of the ceramic beds, he recommends that they not be applied to a furnace if more than 2% salt fluxing is to be used. Although this burner is capable of meeting a NOx limit of 60 ppm in this furnace when operated on natural gas, NOx emissions may be different in other furnaces with other burner/furnace configurations, operating temperatures or other operating conditions. It is suggested the the NOx emission guarantee or 60 ppm, the BACT Part D guideline, be used as a starting point in permitting a new or modified furnace, i.e., in the Permit to Construct, and that the final NOx limit in the Permit to Operate be determined after source testing is completed. Source testing should include the highest-NOx operating mode. |  |  |