Application No.:	  N-2253-15-2

Equipment Category – Flow Coater

| 1. general information |  |  | DATE: 5/13/2003 |
| --- | --- | --- | --- |
| a. manufacturer: Wagner |  |  |  |
| b. type: Sheet Coater |  | c. model: 15X44A |  |
| d. style: With attached curing oven |  |  |  |
| e. applicable aqmd rules: NA |  |  |  |
| f. cost: $ () source of cost data: |  |  |  |
| g. operating schedule: 24 hrs/day 7 days/wk 52 wks/yr |  |  |  |

| 2. EQUIPMENT INFORMATION |  |  | App. No.: N-2253-15-2 |
| --- | --- | --- | --- |
| A. FUNCTION: One of three identical coaters used to apply coatings to sheets used for can manufacture. The facility manufactrures cans to be used for foodstuffs. These coaters apply the coating required for the interior surface of the can. Various coating materials are used depending on the intended contents of the can. |  |  |  |
| b. size/dimension/capacity: 80 sheets/minute (sheets are approx. 3' x 3') |  |  |  |
| c. blowers: |  | d. total flow rate: scfm |  |
| e. material stored/processed/handled: Various organic coating materials (confidential) |  |  |  |
| f. throughput/process rate/usage rate: 80 sheets/minute |  |  |  |

| 3. COMPANY INFORMATION |  |  | App. No.: N-2253-15-2 |  |
| --- | --- | --- | --- | --- |
| A. NAME: Ball Western Can Co. |  |  |  | B. SIC CODE: 3411 |
| C. ADDRESS: 300 W. Gregor Street CITY: Oakdale STATE: CA ZIP: 95361 |  |  |  |  |
| D. CONTACT PERSON: Bob Hall |  | E. PHONE NO.: 303-460-5445 |  |  |

| 4. PERMIT INFORMATION |  |  |  | App. No.: N-2253-15-2 |
| --- | --- | --- | --- | --- |
| A. AGENCY: SJVAPCD |  | B. application type: |  |  |
| C. AGENCY CONTACT PERSON: Mark Schonhoff |  |  | D. PHONE NO.: 209-557-6448 |  |
| E. PERMIT TO CONSTRUCT/OPERATE INFORMATION: P/C NO.: N-2253-15-2 ISSUANCE DATE: 10/7/2001 check if no p/c P/O NO.: ISSUANCE DATE: |  |  |  |  |
| F. START-UP DATE: June 2001 |  |  |  |  |

| 5. EMISSION INFORMATION |  |  |  | App. No.: N-2253-15-2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| A. PERMIT |  |  |  |  |  |  |
| A1. PERMIT LIMIT: Facility has annual, quarterly and daily VOC emission caps. Sheet coaters and curing ovens to be vented to thermal oxidizer. Thermal oxidizer temperature to be maintained at or above value determined in source test to provide 98.5% overall VOC control efficiency. Maximum emissions from oven as lb/MMBtu: NOx-.024 (20 ppmvd@3%O2), CO-0.15, SOx-.00214, PM10-.0076, VOC from gas combustion-.005. Maximum emissions from thermal oxidizer as lb/MMBtu: NOx-.098, CO-.082, VOC-.005, SOx-.00214, PM10-.0076. |  |  |  |  |  |  |
| A2. bact/lAer DETERMINATION: 95% overall VOC control (capture and incineration) |  |  |  |  |  |  |
| A3. BASIS OF THE BACT DETERMINATION: Achieved in practice |  |  |  |  |  |  |
| B. CONTROL TECHNOLOGY |  |  |  |  |  |  |
| B1. MANUFACTURER/SUPPLIER: Smith |  |  |  |  |  |  |
| B2. TYPE: Regenerative |  |  |  |  |  |  |
| B3. DESCRIPTION: The VOC-laden air enters the thermal oxidizer through a bed of heated ceramic material and is thus pre-heated prior to reaching the oxidation chamber. Fuel is injected into the chamber as needed to maintain chamber temperature. A chamber temperature of 1400F was found adequate to achieve the 98.5% destruction efficiency required in the permit. The hot gas leaves the chamber through a bed of ceramic material, transferring most of its heat content to the ceramic material. The direction of flow is periodically reversed by valve actions so that the inlet bed becomes the outlet bed, and visa versa. This oxidizer is rated at 8 MMBtu/hr maximum input, but by retaining most of the heat in the exit bed, little supplemental fuel is needed. Once the chamber is heated to temperature, the VOC content of the inlet air provides sufficient heat, and no supplemental fuel is required. |  |  |  |  |  |  |
| B4. CONTROL EQUIPMENT PERMIT APPLICATION DATA: P/C NO.: ISSUANCE DATE: P/O NO.: ISSUANCE DATE: |  |  |  |  |  |  |
| B5. WASTE AIR FLOW TO CONTROL EQUIPMENT: FLOW RATE: 36,000 scfm ACTUAL CONTAMINANT LOADING: 1000 ppmv (approx.) BLOWER HP: |  |  |  |  |  |  |
| B6. WARRANTY: |  |  |  |  |  |  |
| B7. PRIMARY POLLUTANTS: VOC |  |  |  |  |  |  |
| B8. SECONDARY POLLUTANTS: NOx, CO, VOC, PM10 |  |  |  |  |  |  |
| B9. SPACE REQUIREMENT: |  |  |  |  |  |  |
| B10. LIMITATIONS: |  |  |  |  |  | B11. UNUSED |
| B12. OPERATING HISTORY: The oxidizer has been in regular service since startup. |  |  |  |  |  |  |
| B13. UNUSED |  |  | B14. UNUSED |  |  |  |
| ### C. CONTROL EQUIPMENT COSTS |  |  |  |  |  |  |
| C1. CAPITAL COST: check if installation cost is included in capital cost EQUIPMENT: $ INSTALLATION: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| C2. ANNUAL OPERATING COST: $ () SOURCE OF COST DATA: |  |  |  |  |  |  |
| D. DEMONSTRATION OF COMPLIANCE |  |  |  |  |  |  |
| D1. STAFF PERMFORMING FIELD EVALUATION: ENGINEER'S NAME: INSPECTOR'S NAME: DATE: |  |  |  |  |  |  |
| D2. COMPLIANCE DEMONSTRATION: Initial source test for overall VOC control efficiency, and annual source test for VOC destruction efficiency. |  |  |  |  |  |  |
| D3. VARIANCE: NO. OF VARIANCES: DATES: CAUSES: |  |  |  |  |  |  |
| D4. VIOLATION: NO. OF VIOLATIONS: DATES: CAUSES: |  |  |  |  |  |  |
| D5. MAINTENANCE REQUIREMENTS: |  |  |  |  | D6. UNUSED |  |
| D7. SOURCE TEST/PERFORMANCE DATA RESULTS AND ANALYSIS: DATE OF SOURCE TEST: 2/9/2002 CAPTURE EFFICIENCY: Permanent Total Enclosure criteria met DESTRUCTION EFFICIENCY: >99% OVERALL EFFICIENCY: >99% SOURCE TEST/PERFORMANCE DATA: OPERATING CONDITIONS: TEST METHODS: U.S.EPA Method 25A for VOC destruction efficiency, U.S.EPA Method 204 criteria for Permanent Total Enclosure |  |  |  |  |  |  |

| 6. COMMENTS |  | App. No.: N-2253-15-2 |
| --- | --- | --- |
| Permit to Operate is in process. This sheet coater coats steel sheets for manufacture of "three-part" cans, such as those used for food products. Relatively high-VOC (40-70%) coating materials are used for this type of can. |  |  |